Five lessons from Henry Ford Leading to a Framework for Business to Business E-Production

George Collier, Paolo Missier (Applied Research Bellcore) and Dennis Shasha (Courant Institute, NYU)

Imagine a world in which a company C sets itself up as a single point of contact for a collection of telephony, internet, and product services. A customer contacts C, specifies his or her desires, and gets billed by C. C in turn marshals services and physical resources to be allocated at the best possible tradeoff among price, quality, and delivery date to satisfy those desires. How should such an organization be constructed and what computer support is necessary? We argue that both the positive and negative historical experiences of Henry Ford suggest an architecture supporting multi-enterprise flexible but controllable flows. We explain the reasoning for this conclusion and the resulting virtual enterprise architecture in some detail.

# Introduction.

Fundamental human economic activity can be divided into three spheres: production, marketing, and consumption. Production is the creation of a good or service. Marketing is the buying and selling of the goods produced. Consumption is their utilization. Much of the recent interest in electronic commerce has focused on the latter two spheres, while the first has been relatively neglected. In this paper, we present our initial work towards the definition of a general framework for supporting E-Production. Reflecting our emphasis on marketing and production, the main components of our design consist of a constraint-based system for eliciting customer’s desires, a production planning system to find a good strategy for satisfying them, and a workflow management system for carrying out that strategy.

The pace, change and the rapid expansion of the Internet and wide area networking has created a daunting challenge to those who would attempt to predict the evolution of E-Production. We have found it useful to think of E-Production in a historical context. We present this perspective in the first part of the paper, and then use it to introduce our proposed architecture.

# 1.	Historic Motivation

Mass production is the focussing upon a manufacturing project of the principles of power, accuracy, economy, system, continuity, and speed. Henry Ford in Enc. Britannica (1926)

## 1.1.	Increasing Command Over Production Processes

One of the clearest historical trends in production is the increasing ability of human beings to control the processes, evolving from similarity. For example, early stone tool production was characterized by extreme variability, to the point where anthropologists are left to argue whether the tools can be differentiated into types and what their ultimate purpose was. After some number of hundreds of thousands of years, stone tool production had advanced to the point where artisans could reliably produce and reproduce tools of specific types.

By the 1700s armies depended on the complex technology of guns. These weapons were fragile and tended to break in the field. Interchangeable parts would, French military planners realized, support repair in the field by non-experts. Interchangeable parts became a goal of French at the turn of the 18th century and then a goal of the US military through the influence of French officers.

The goal of producing interchangeable parts remained elusive for almost 100 years; achieved rarely and only at great cost (8). However, Henry Ford made interchangeable parts a criterion of mass production: “In mass production there are no fitters” (49). Success required the application of the techniques of mechanical automation and special purpose machines. We see this development as fundamentally an achievement in the arena of automation and process control. Interchangeability of parts reduces to the repeatability of a process and the ability to meet a design specification; which imply a fine control over the production process itself.

This was achieved by the use of a system of fixtures, jigs (which correctly position a piece for an operation), gauges (which measure the difference between a part and its ideal form) and special purpose machines (6). The engineer who first designed the jigs, fixtures and gauges for the Ford factory called them “farmer tools” since, he claimed, they could be used by farmers to produce work as good as a skilled mechanic (221).

Part interchangeability was one of the first critical steps towards the revolution to be wrought by the assembly line. This, in many ways, was the culmination of the trend of increasing control over production processes.

Lesson I: Efficient production and assembly demands fine control over process and interchangeable subcomponents.

## 1.2.	Specialization of Purpose

A key contributor to Ford’s success in producing interchangeable parts was the application of special purpose machines which automated many of the tasks formally performed by skilled workman. Much ingenuity was applied to the task of inventing machines that could perform very specific steps in the production process with great efficiency and accuracy while demanding little from their operators. For instance, a machine was designed that drilled at one time forty-five holes in four sides of the block. It was pointed out that “these spindles are non-adjustable so far as location is concerned” (233). The Ford engine tools provided examples “of the single purpose machine carried to the limit. (233)”.

Lesson II: efficient production often requires the development of machines (or people) that specialize in a particular type of labor.

## 1.3.	Flow Production

Early in the production of predecessor of the Model T, the Model N, one of Fords engineers rearranged the factory so that machine tools were placed according to the sequence of operations to be performed, rather than by type of machine (221). An observer of the factory built to make the Model T described it thusly:

“So thoroughly is the sequence of operations followed that we not only find drilling machines sandwiched in between heavy millers and even punch presses, but also carbonizing furnaces and babitting equipment in the midst of the machines.” (229)

Organization by flow or process was carried even more thoroughly by the application of the principle of “moving the work to the men” (237) or as Ford himself termed, “flow production” (241). According to this principle every thing was put into motion and every man stood still. For instance, one of the first assemblies to which this technique was applied was the flywheel magneto. Workers stood beside a long, waist-high row of flywheels that rested on smooth, sliding surfaces on a pipe frame. They were instructed to place on particular part in the assembly or perhaps to start a few nuts and then push the flywheel down. Having pushed the flywheel down, the worker then repeated the same step over and over again. The time to assemble a magneto dropped from 20 minutes a worker to 13 minutes. With further refinement, including adjusting the height of the line, adding a chain drive and lowering the number of workers, assembly time was cut to 5 minutes – a gain of 300% in productivity (247).

Given these successes, the principles of flow production were applied throughout the factory and the first mass production facility was created. Underlying the design of flow production were “operations sheets” which described all the machine tool operations for each part and the type and number of machines required for each operation. Operations sheets were maintained for assemblies as well, listing step-by-step procedures, tools required and special problems that might be encountered (271). These then, were the first workflow specifications.

Lesson III: effective use of the specialized machines that perform the work requires their coordination through workflow modeling, design and enactment control.

1.4.	Production and Flexibility

Henry Ford not only invented mass production, his company became the first “victim” of it. Ford’s Model T quickly dominated the market for the inexpensive automobile which, in some sense, it created it by lowering the price point to a level where the “masses” could afford one. However by the late 1920s Ford faced a dramatic decline in his market share. In the early 1920s Ford had 50% of the market, however, by 1927 Ford had only 15% (276).

This was due to the “changeless” nature of the Model T. By the mid-1920s the Model T was outdated. Genuine engineering advances had occurred, e.g., battery powered ignition, electric starters, and shock absorbers (276). The underlying problem was the inflexibility of the assembly lines and factories that Ford had built. At General Motors, designers had realized the need of the public for novelty. The assembly lines at Ford were supremely good at producing one thing and one thing only: Model Ts. However, technology advanced and, even more important, people grew bored with the same product, year after year. GM, realizing this, had moved to production techniques that allowed yearly superficial changes to the cars and the occasional more deep change. This gave GM an edge on Ford.

Years later, in the 1980s a similar drama was played out, as Japanese auto manufacturers captured large shares of the automotive market from traditional producers. Once again, nimble, flexible producers where able to overcome larger more inflexible producers.

The exquisitely fine grained control over the production process associated with mass production brings with it a blessing and a curse. The blessing is that the reduction in cost and time taken to produce an item. The curse is: to the degree that the system is optimized for making a particular item, it will be difficult to change. Specialists have trouble adapting to changing conditions. The dilemma for a producer is to achieve efficiency through techniques such a automation while not sacrificing too much flexibility.

Lesson IV: the technology (mechanical and human) applied to control the process must be flexible.

## 1.5.	Vertical Integration vs. Supplier Chain Management

A vertically integrated enterprise owns and controls all the stages of production. At River Rouge, in Detroit, Henry Ford built a fantastic monument to vertical integration:

“the complex included virtually every element needed to produce a car: blast furnaces, an open hearth mill, a steel rolling mill, a glass plant, a huge power plant and, of course, an assembly line. Ninety miles of railroad track and miles more of conveyor belts connected these facilities, and the result was mass production of unparalleled sophistication and self-sufficiency.” (http://www.nps.gov/crweb1/nr/detroit/d38.htm)”

Eventually Ford even owned iron mines and rubber plantations to supply his factory (MTCW 39). Years later, we now recognize that this approach to production as wrong because it is hard for one company to have a comparative advantage in everything, and an automotive company relies on between 300 (Japanese model) to 2000 (US model) suppliers. Supplier integration varies between 30% (GM in the 1980s) to 75% (Saab in 1980s) (MTCW 139). Efficient manufactures tend to rely on fewer suppliers which provide them with larger components (e.g., a complete seat assembly – Infiniti -- rather than 25 suppliers making 25 parts – GM pg 146). These suppliers are tightly coupled into the assembler’s operations, to the extent of working with the assemblers designers on a daily basis.

Lesson V: production chains should span companies to ensure	that each part of the process is done by the company having the greatest competence.

# 2.	A framework for E-Production.

These five lessons provide us with guidelines for the design of an architecture to support multi-enterprise business. Namely, the architecture should provide support for:

- flow modeling and design,

- multi-company production chains, with job allocation based on competence and specialization (of machines, or processes),

- fine-grained process control, and

- flexibility in accommodating technology advances.

These basic requirements translate into a framework for E-production that covers the three main steps of an economic activity:

- Marketing: Customer determines the product desired, using a constraint-based (expert) system;

- Production:

- a Planner says how to produce the item,

- a Workflow system produces it;

- Consumption: Customer receives it.

Figure x: Customer interacts with a database-sensitive expert system.

The full architecture extends the lessons suggested by Ford towards the customer. Thus, the first step is to elicit the desires of the customer. These desires must be balanced by what can be produced. Thus, the top box in figure x represents a module that suggests products to customers along with pricing and schedule information (information derived from the planner). The user should be able to specify the desired product exactly or through criteria such as color, weight, functionality, price, quality, delivery date and so on. Some of these criteria may be hierarchically related in a kind-of hierarchy, e.g. ISDN and POTS are both kinds of telephonic functionality. More complicated relationships are also possible, e.g., it makes sense to ask about loss rates only if one is renting bandwidth. We imagine that the user will specify his choices by descending a directed graph.

The constraint-based system which elicits the customers desires is “database-sensitive”. A database-sensitive system changes its output seamlessly as the database upon which it is based changes. Thus, the expert system is sensitive to changes in the quality or price of components. The database-sensitive planner produces a plan satisfying the price, quality and delivery constraints of the customer. This must be database-sensitive in the same way (in this case, a database of available products and production methods). A database-sensitive workflow system produces the products using the notions of long transactions.

The middle tier has to do with figuring out how to produce the product. This tier must do a constrained optimization among several criteria such as quality, price, schedule, quantity producible, and perhaps even side effects (e.g. pollution caused). A slightly simplified data model for this part consists of modeling components (either products or parts of products) and their inventory, purchases (what can be bought at what price and with what virtues), and processes (what can be produced and how).

## 2.1.	Schema for Planner

In this section we present the schema for the planner.

| Relation | Comments |
| --- | --- |
| component([id], desc, numavailable, price, defects) |  for the number available, we have a price and defect rate on average. |
| purchase([id], [componentid, supplier, purchasemin, purchasemax], price, deliverDate, defects) |  e.g. price may differ according to quantity (function of qty in general)  adding purchasemin, purchasemax to the key indicates  that price etc. can be a function of quantity  They are the minimum and maximum quantity one can purchase at the price. |
| process([procid] supplierid, price, timetodo, defects, maxinstances) |  procid is the process identifier.  supplierid is the identifier of the supplier.  timetodo is the time it takes to do it.  maxinstances is the maximum number of instances of the process  that can be instantiated.  This puts an upper limit on what can be produced. |
| processResult([procid, componentid], qty) |  a process produces a component  qty indicates the # of result components produced  by each instance of the process. |
| processSupply([procid, componentid], qty) |  qty indicates the # of supplyComponents needed to do  each instance of the process |

Figure y: Schema for the planning component has five input tables reflecting components, purchases, and processes, and one output table, namely the set of interesting plans. Defining what is interesting and finding those plans requires a planning algorithm which can be either exhaustive or heuristic.

This dependent table gives us several ways to create components. Each record includes the partial order of processes or purchases required to construct the target component. Each process has an associated costperunit, deliverDate, defects. qty producible is the number that can be produced. starttime, finishtime are vectors corresponding to the start and finishtime of each process in the plan. plan([componentid, idvec (procid or purchaseid vector), idcount, starttime, finishtime] costperunit, deliverDate, defects, qtyproducible)

## 2.2.	Enactment

The planner produces a plan expressed in a workflow specification language (e.g., FDL). The workflow component carries out a plan. The workflow engine is connected (via the network) to internal and external systems. If all plans execute without problem, this is straightforward. However, if workflows can run into unexpected delays, there is a feedback loop back into the planner at which point a different plan must be tried.

# 3.	An Example

Imagine an executive who wants a telecom/data package which includes: 1) cell phone (calling plans ...) 2) high speed data line to apartment. 3) stock market data feeds, other information feeds 4) long distance service (calling plans) 5) email to cell phone 6) pots phone 7) internet access. The executive interacts with our system (elicitation) which steps her through the various options refining the product description as she goes.

Then the planner determines that AT&T should provide the cell phone, Qwest should supply the high speed data line, Reuters should supply the stock feeds etc. This plans is based on a database of prices and availability.

We consider that optimization has a planning component: For example, suppose the customer wants service from a New Jersey site to a California site. Suppose Qwest can offer transport from a New Jersey office to a California office, then for this service to be of interest, C must contract the service from the customer site to the New Jersey office and from the California office to the customer's second site.

Further, buying several services from the same supplier may entitle one to a discount.

So, the provisioning system, like a configuration system, must solve an optimization problem across a variety of vendors and products.

# 4.	Conclusion

We have argued on historical grounds that future production systems must meet five criteria and we have described a framework to meet them:

- Process control: achieved in our workflow system;

- Specialization of effort: suppliers provide this;

- Flow modeling, enactment and management: : planner constructs a model and workflow system carries it out;

- Flow flexibility: the flow is constructed dynamically in a database-sensitive manner

- Cross company boundaries: our database makes no distinction between intra-company and extra-company processes;

We have described a framework and architecture for a system which directly addresses these criteria. We are implementing a prototype of this design.